# STANDARD OPERATING PROCEDURE

## Embryo and Blastocyst Grading

PRINTED COPIES OF THIS DOCUMENT ARE UNCONTROLLED PLEASE REFER TO Q PULSE FOR THE LATEST VERSION

| DOCUMENT DETAILS |  |  |  |
| --- | --- | --- | --- |
| AUTHOR (NAME & JOB ROLE): |  |  | Lyndsey Zujovic, Group Director of Embryology |
| DOCUMENT APPLICABLE TO: |  |  | TFP GROUP (ALL) |
| DISTRIBUTION GROUP: |  |  | All Laboratory Staff |
| AMMENDMENT HISTORY |  |  |  |
| REVISION | ACTIVE DATE | CHANGES MADE |  |
| 1.0 | 28/01/2020 | Initial release. |  |
| 2.0 3.0 | 20/05/2021 18/05/2022 | Section 7 updated with details of the EQA scheme. Embryos graded E/F to be cultured until Day 6 as standard with a view to potential cryopreservation. Culture to Day 7 at local clinics discretion. |  |

#### Purpose

To describe the time points at which embryos are graded and the associated grading schemes used to grade embryos. To define the process of embryo grading at cleavage stage and Day 5/6 of development.

#### Scope

Applicable to all laboratory staff deemed competent to perform embryo grading.

#### Definitions

TL		Time Lapse

EQA		External Quality Assurance

PTMRIE		Polish Society for Reproductive Medicine and Embryology

#### Responsibilities

            - General Manager

The General Manager (GM) is accountable for the implementation of this clinic-wide process.

            - Laboratory Manager (Clinic)

The Laboratory Lead is responsible for ensuring that current practice is line with this SOP, all staff have been trained appropriately and signed off competent to enable them to work safely and ensure patients safety at all times. The Laboratory Lead is also responsible for ensuring that all non-compliance with this SOP, or incidents occurring are properly investigated and any corrective action taken. They are also responsible for auditing compliance to this SOP.

            - Quality Manager

The Quality Manager is responsible for ensuring that all staff are aware of and comply with the requirements specified in this document. The QM is also responsible for ensuring all non-compliance with this SOP, or incidents occurring are properly investigated and any corrective action taken is recorded on QPulse.

            - All Staff

All staff re responsible for following the process laid out in this document, ensure they receive up-to-date training to enable safe working and to request further training if they feel it is required. Staff are also responsible for reporting any non-compliance or incidents occurring on QPulse and following up the outcome of any reported incidents/non-conformities.

#### Procedure

##### 5.1	Equipment & Consumables Required

Equipment/Consumables Required

TL Incubator (ES+ or Geri)

Stereo/Inverted Microscope

Dishes/Media Required

N/A – Embryos are already in their culture dishes at this stage

##### 5.2	Equipment Checks

Prior to observing embryos, the equipment to be used must be checked to ensure it is operating optimally.

The flowhood must be checked to ensure the flow is on and the hood is clear of used dishes/pipettes and tubes.

Ensure the RI heated stage/Inverted or Stereo Microscope is switched on and at the correct set point.

Ensure the embryos in the wells of the TL dish are clearly visible.

##### 5.3	Timing of Embryo Grading Assessments

Table 1: Timing of embryo grading observations and which features should be assessed.

|  | Time Lapse Patients TBC | Standard Culture |
| --- | --- | --- |
| Day 2 | Cell No. + Grade Note the presence of vacuoles/granularity/reverse cleavage/direct cleavage/Multi Nucleated Cells | No Observation unless high risk of failed cleavage/cleavage stage embryo transfer planned |
| Day 3 | Cell No. + Grade Note the presence of vacuoles/granularity/reverse cleavage/direct cleavage/Multi Nucleated Cells | Cell No. + Cohort Quality |
| Day 4 | No Observation Required | No Observation Required |
| Day 5 / 6 | Stage/Grade KID Score Note the presence of Excluded cells /Degenerate cells/vacuoles/granularity/’strings’ | Stage/Grade Note the presence of Excluded cells /Degenerate cells/vacuoles/granularity/’strings’ |

Detailed information describing the grading of embryos cultured in time lapse can be found in the TFP Time Lapse SOP (TFP1-SOP-EMB-0022).

##### 5.4	Embryo Grading Procedure

For standard culture, consideration must be given to maintaining temperature, pH and osmolality during the embryo grading procedure, therefore oil covered dishes are used.  The procedure should be performed as quickly as practically possible to minimise the exposure of the embryos outside the incubator environment.

Embryo grading must be performed at x200-x400 magnification in order to be able to observe the necessary features of the embryo to allocate a grade accurately.

The aim for all patients is to culture embryos until Day 5 and perform embryo transfer at that stage.

Not all patients will have embryos suitable for transfer on Day 5 and patients must therefore be counselled about this risk of cancellation (on average approximately 5%).

Embryos that are cleavage stage on Day 5 or 6 are not suitable for embryo transfer.

In the event that no embryos are suitable on Day 5 i.e. the embryos are all still at cleavage stage, either cancel the embryo transfer or culture until day 6 and reassess embryo development at that stage.

In the event that the patient has a low number of embryos, or the embryo quality is low, a discussion must be had with the patient explaining the benefits and risks of day 5 culture.  The embryologist and patient must then decide the most suitable way forward.

For detailed grading on Day 2 or 3 i.e. for TL patients or patients having a Day 2/3 embryo transfer, use the grading scheme below to grade the embryos.

Note the cell number alongside separate grades for cellular regularity and degree of fragmentation.  Cellular regularity and fragmentation are graded on a scale of 1-4 with a grade 4 denoting the highest quality embryo.

e.g.	5/4/2		5 cell, regular cells, 20-50% fragmentation

Regularity is graded in a stage specific way i.e. a 3-cell embryo with one large cell and two small cells would be regarded as ‘regular’.

Also note the presence of any vacuoles, granularity, reverse cleavage, direct cleavage and multi-nucleated cells.

Table 2: Description of the Day 2/3 TFP Grading Scheme

| Cell No. | Grades | Blastomere Regularity (Stage Specific) | Fragmentation |
| --- | --- | --- | --- |
| Cell Number | 4 | All Cells Regular | <10% |
|  | 3 | Majority of Cells Regular | 10-20% |
|  | 2 | Majority of Cells Irregular | 20-50% |
|  | 1 | All Cells Irregular | >50% |

Table 3: Description of the Day 5/6/7 TFP Grading Scheme

| Grade | Morphological Features |
| --- | --- |
| A | Expanded blastocyst with thinned ZP & cavity >50% of the embryos volume, with a clear ICM. TE consists of many cells and is cohesive. |
| B | Full Blastocyst but not yet expanded with a clear ICM. TE consists of many cells and is cohesive. |
| C | Blastocyst with a small ICM and/or TE consisting of fewer cells and is loosely cohesive. |
| D | Blastocyst with no ICM and/or TE consists of very few cells and is non-cohesive. |
| E | Any sign of cavitation but ICM and TE not yet visible |
| F | Any sign of compaction |
| Cleavage | Separate cells clearly visible and no sign of compaction |

When selecting embryos for transfer, preference must be given to the embryo(s) with the highest grade and those on target in terms of their development.  For clinics using TL, KID scores for example should also be considered.

Embryos with correct number of cells for their age with regularity and fragmentation scores of 3 or 4 are considered suitable for cryopreservation on Day 2/3.

Embryos graded A, B or C on Day 5/6 are considered suitable for cryopreservation.

Embryos graded E-F on Day 5 should be cultured until Day 6 and then re-assessed for suitability for cryopreservation.  Culture to and cryopreservation on Day 7, is at the local clinic’s discretion.

Note: In Poland embryos graded D may also be considered for cryopreservation. In addition, the PTMRIE grade must also be recorded in the laboratory notes.

All clinics must participate in the TFP EQA scheme and review their clinical practice in line with the results.

#### Standards/Key Performance/Quality Indicators

% Patients having Day 5 Embryo Transfer 	≥90%

% Blastocyst Formation Day 5 			≥40%

% Grade A or B Blastocysts Day 5 		≥30%

% Gold Standard Match on EQA 		              ≥70%

#### Monitoring

The TFP External Quality Assurance (EQA) scheme is distributed three times per year and assesses individual operators’ agreement with the gold standard grade.  Any operators with <70% agreement are required to regrade the embryos following discussion with their laboratory manager.

Results of the TFP EQA scheme are discussed at each quarterly Head of Laboratories meeting.

A TFP Laboratory Manager is required to audit this SOP annually.