# STANDARD OPERATING PROCEDURE

## Oocyte Vitrification – Vitrolife

PRINTED COPIES OF THIS DOCUMENT ARE UNCONTROLLED PLEASE REFER TO Q PULSE FOR THE LATEST VERSION

| DOCUMENT DETAILS |  |  |  |
| --- | --- | --- | --- |
| AUTHOR (NAME & JOB ROLE): |  |  | Lyndsey Zujovic |
| DOCUMENT APPLICABLE TO: |  |  | TFP GROUP (ALL) |
| DISTRIBUTION GROUP: |  |  | ALL LABORATORY STAFF |
| AMMENDMENT HISTORY |  |  |  |
| REVISION | ACTIVE DATE | CHANGES MADE |  |
| 1.0 | 28/01/2020 | Initial release. |  |
| 2.0 3.0 4.0 | 10/05/2020 01/10/2021 08/01/2023 | Oil overlay added to dish (excluding Vitri 3) Witnessing updated to include the use of three identifiers. Two droplets of Vitri 3 increased in size to 50µl in line with manufacturers guidelines Section 6 extended to include additional KPI’s. |  |

#### Purpose

To describe the procedure for vitrifying oocytes using the Rapid I device with Vitrolife vitrification kits.  Oocyte cryopreservation can be used to preserve female fertility or to store oocytes for later use in an oocyte donation programme. The reasons for oocyte vitrification may include fertility preservation for oncology patients or ‘social egg freezing’ to allow women to preserve their fertility prior to age-related fertility decline.

#### Scope

Applicable to all Embryologists and practitioners deemed competent to carry out vitrification of oocytes:

Gamete/embryo donation and surrogacy are prohibited in Kuwait, as such any references to this are not applicable to clinics in Kuwait.

#### Definitions

PPE		Personal Protective Equipment

#### Responsibilities

            - General Manager

The General Manager (GM) is accountable for the implementation of this clinic-wide process.

            - Laboratory Manager (Clinic)

The Laboratory Lead is responsible for ensuring that current practice is line with this SOP, all staff have been trained appropriately and signed off competent to enable them to work safely and ensure patients safety at all times. The Laboratory Lead is also responsible for ensuring that all non-compliance with this SOP, or incidents occurring are properly investigated and any corrective action taken. They are also responsible for auditing compliance to this SOP.

            - Quality Manager

The Quality Manager is responsible for ensuring that all staff are aware of and comply with the requirements specified in this document. The QM is also responsible for ensuring all non-compliance with this SOP, or incidents occurring are properly investigated and any corrective action taken is recorded on QPulse.

            - All Staff

All staff re responsible for following the process laid out in this document, ensure they receive up-to-date training to enable safe working and to request further training if they feel it is required. Staff are also responsible for reporting any non-compliance or incidents occurring on QPulse and following up the outcome of any reported incidents/non-conformities.

#### Procedure

##### 5.1	Equipment & Consumables Required

Equipment/Consumables Required

- Serological pipettes

- Rapid I devices

- Heat sealer

- Rapid I Liquid Nitrogen Box

- Liquid Nitrogen

- Pre pulled Pipettes (145-190µm)

- Timers

- Pipette and tips

- Cane/Cryocassette/Visi

Dishes/Media Required

- 5 well dishes

- ICSI dishes

- Rapid Vit Oocyte Media

- Oil

- RI Witness Tags

##### 5.2	Equipment Checks

Prior to the vitrification procedure the equipment to be used must be checked to ensure it is operating optimally.

The flowhood must be checked to ensure the flow is on and the hood is clear of used dishes/pipettes and tubes.

Ensure the RI heated stage is switched on and at the correct set point. Vitrification is performed at 37◦C.  Maintenance of a temperature of 37◦C is critical to the success of the procedure.

Fill the Rapid I Cryobox and a dilvac with liquid nitrogen.  Ensure appropriate PPE is used when handling liquid nitrogen.

Ensure the heat sealer is closely situated and switched on.

##### 5.3	Witnessing Checks (Full Name, Clinic Number and D.O.B)

Before beginning the Vitrification process check the laboratory notes to ensure that ‘Oocyte Vitrification’ or ‘Oocyte Donation with Vitrification’ is the agreed treatment.

Check the required HFEA forms have (or country specific equivalent) been completed providing consent to oocyte freezing.

Oocyte vitrification requires a second witness to be used alongside RI witness.

For vitrification of the patients own oocytes, check the patient’s full name, clinic number and D.O.B. on the G1+ Post Denudation dish and vitrification dishes matches that on the laboratory notes and RI witness system.  Check the patient’s full name, clinic number, D.O.B. and date of vitrification on the Rapid I straws and visi/cassette match that on the laboratory notes and RI witness system.  Each straw must be labelled with the number of oocytes contained within.  Approve these checks on the RI witness system and both operator and witness must sign for these checks.

For vitrification of donor oocytes, check the full name, donor number and D.O.B.  on the G1+ Post Denudation dish and vitrification dishes match that on the laboratory notes and RI witness system.  Check the donor number, IDEAs Treatment Cycle ID and date of vitrification on the Rapid I straws and visi/cassette match that on the laboratory notes and RI witness system.  Each straw must be labelled with the number of oocytes contained within.  Approve these checks on the RI witness system and both operator and witness must sign for these checks.

##### 5.4	Preparation of Dishes

Prepare the oocyte vitrification dishes 30-60 minutes prior to use (validate time period required locally) and allow to warm in an ungassed incubator to 37◦C.  Prepare one oocyte vitrification dish per 3 Rapid I’s to be used.  One ICSI dish is also required for every oocyte vitrification dish.

Gently invert the Rapid Vit Oocyte solutions 3 times prior to making the dish.  Place 0.8ml of Vitri1, 0.8ml of Vitri2 and 0.8ml of Vitri3 in separate wells of a 5 well dish.  Overlay Vitri 1 and 2 with 0.3ml of oil.

Vitri 1	 +Oil							Vitri 2 + Oil

Vitri 3

Figure 1: Vitrification dish set-up

##### 5.4	Timing of Vitrification Procedure

Perform oocyte vitrification 3 hours post oocyte collection ± 30 minutes.

##### 5.5	Cryoprotectant Exposure Times

Vitrolife Rapid Vit Oocyte

Vitri1: 	Exposure time 5-20 minutes

Vitri2: 	Exposure time 2-5 minutes (until oocyte has re-expanded back to its original size, usually around 4 minutes)

Vitri3: 	Exposure time 35 seconds

##### 5.6	Preparation for the Procedure

Label sufficient Rapid I’s for the number of oocytes to be vitrified.

1-3 oocytes can be placed on each Rapid I.

Identify a location for the samples within the cryostore based on the patient’s infection screen results.  Split fertility preservation patient’s samples between a minimum of two dewars.  Donor Oocytes must be placed in a quarantine dewar.

Enter the storage location(s) in IDEAs. Prepare a storage card if required.

##### 5.7	Procedure

Maintenance of a temperature of 37◦C is crucial during oocyte vitrification.  Temperature can be maintained using either a heated stage or an ungassed G85/incubator.

Using a 145-190µm pipette transfer the number of oocytes to be placed on up to 3 Rapid I’s into Vitri1.  Immediately start the timer for 5 minutes.

Wash the oocytes within Vitri1, replace the lid on the dish and culture at 37◦C for 5 minutes.

Place the Rapid I’s for this round into the Rapid I cryobox to cool.

After 5 minutes, prime the pipette with solution from Vitri2.  Collect the number of oocytes to be placed on one Rapid I and transfer into Vitri2.  Immediately start the timer for 5 minutes.  Wash the oocytes in Vitri2.

Oocytes must remain in Vitri2 for 2-5 minutes during which time they should re-expand.  Once re-expanded they can be transferred into Vitri3.

Immediately before transferring the oocytes into Vitri3 make 2 x 50µl drops of Vitri3 within the ICSI dish.  Remove the metal rod from within the Rapid I.

Prime the pipette with solution from Vitri3.

Transfer the oocytes into the drops of Vitri3 in the ICSI dish.  Empty the pipette and wash thoroughly in both wash drops.

Place the oocytes on the Rapid I.  Immediately plunge within the Rapid I straw.  The total duration in Vitri3 must not exceed 35 seconds.

Seal the Rapid I straw with the heat sealer.

Repeat for the remaining oocytes in the dish.

Once all oocytes have been vitrified within the dish, repeat the whole process until all the oocytes have been frozen.

Place the oocytes into a storage dewar alongside a second witness.

Clear the flowhood of any used consumables.  Use Oosafe spray to clean the flowhood surface.

Complete the laboratory and witnessing records.

#### Standards/Key Performance/Quality Indicators

Oocyte Survival 						≥90%

2PN Rate						≥55%

Blastocyst Formation Rate (A-E on day 5)			≥45%

Good Blastocyst Formation Rate (A/B/C Day 5 or 6)	≥25%

KPI’s with donated oocytes are expected to meet the lower limits above.  KPI’s with own oocytes may be lower depending on patient age, patient history.

#### Monitoring

#### A TFP laboratory manager is responsible for auditing this SOP once annually.

#### Each laboratory manager is responsible for monitoring the operator’s oocyte survival rates within their clinic.  Any deviation will be discussed at team meetings or supervision meetings.